MAVLink/ArduCopter Fuzzing Study – ICES Relevance Summary

One-page technical summary of the paper and current SITL test results

Overview

The paper investigates the robustness of ArduCopter against malformed and unexpected MAVLink inputs using a dedicated fuzzing framework. Its approach systematically generates random, boundary-value, and structurally malformed messages, sends them to ArduCopter SITL, records reproducible test cases, and observes failures such as crashes or abnormal behavior. The original work used an older MAVLink/SITL environment; our implementation modernizes the approach for the current ArduCopter SITL and MAVLink 2.

Why this is relevant to the ICES program

Yes — this testing is directly relevant to ICES because ICES depends on reliable UAV-to-ground and airborne communication. MAVLink is a control/telemetry protocol commonly used around autopilot systems, so testing how an autopilot handles malformed or unexpected communication inputs helps assess the robustness of a UAV node at the communication interface. For ICES, this provides a security-testing foundation for later evaluating communication resilience, fault handling, and protocol robustness in a controlled laboratory environment.

Current experimental status

The current work has reproduced the paper’s seven test categories in a modernized MAVLink 2/ArduCopter SITL setup. The completed cases below did not produce an observed SITL crash. Case 2 requires a corrected full rerun because the first run encountered TCP backpressure/timeouts caused by the test harness sending data faster than SITL consumed it; this was not observed as a SITL crash.

| Case | Test condition | Current result |
| --- | --- | --- |
| 1 | Completely random data | No crash detected |
| 2 | Message IDs + random payload lengths | Corrected full rerun remaining |
| 3 | Zero-byte payload | 256/256; no crash |
| 4 | All 00 payload; lengths 1–255 | 255/255; no crash |
| 5 | All FF payload; lengths 1–255 | 255/255; no crash |
| 6 | Random payload; lengths 1–255 | 255/255; no crash |
| 7 | Declared length differs from actual by -1 / +1 | 509/509; no crash |

Conclusion / remaining work

The results provide an initial indication that the tested malformed-input classes did not crash the present ArduCopter SITL configuration. However, this is not yet a complete robustness assessment. The full corrected Case 2 run remains outstanding, and broader testing is still required, including higher-volume execution, additional message types/fields, parser-state and sequence interactions, transport conditions, logging/diagnostics, and reproducibility of any abnormal result. Therefore, the current outcome should be treated as preliminary security-testing evidence for ICES rather than a final robustness certification.