Fuzzing Test Case Conditions and Results

ArduCopter SITL / MAVLink Test Matrix

Summary: The table below describes each fuzzing test case, what the case sends to ArduCopter SITL, and the current execution result.

Test-case descriptions: Each case changes a specific part of the MAVLink input so that the effect of that condition can be observed while keeping the remaining test setup controlled.

| Case | Test condition | Result | What the test does |
| --- | --- | --- | --- |
| 1 | Completely random data | No crash detected | Generates completely random raw bytes with lengths from 1 to 1000 bytes and sends them to the SITL TCP endpoint. This tests how the receiver behaves when the incoming data is not intentionally constructed as a valid MAVLink frame. |
| 2 | Message IDs + random payload lengths | Needs corrected full rerun | Iterates through message IDs 0-255 while increasing the payload length from 1 to 255 bytes and using randomized payload bytes. Each input is built as a MAVLink 2 frame to exercise many message-ID/payload-size combinations. |
| 3 | Zero-byte payload | 256/256, no crash | Sends MAVLink 2 frames for message IDs 0-255 with a payload length of exactly zero bytes. This checks how the parser handles frames that contain no payload data. |
| 4 | All 00 payload | 255/255, no crash | Uses message ID 0 and sends payloads containing only 0x00 bytes, increasing the payload length from 1 to 255 bytes. This isolates the effect of an all-zero payload pattern across different payload sizes. |
| 5 | All FF payload | 255/255, no crash | Uses message ID 0 and sends payloads containing only 0xFF bytes, increasing the payload length from 1 to 255 bytes. This tests the parser with the maximum byte value repeated across different payload sizes. |
| 6 | Random payload, increasing length | 255/255, no crash | Uses message ID 0 with random payload contents and increases the payload length from 1 to 255 bytes. Unlike Cases 4 and 5, the payload changes randomly while the MAVLink 2 frame structure and CRC are constructed for each test. |
| 7 | Declared length +/-1 from actual | 509/509, no crash | Deliberately makes the MAVLink 2 header length disagree with the number of payload bytes actually sent. Both directions are tested: declared length one byte smaller (-1) and one byte larger (+1) than the actual payload. |